<Device API>

| - S7, X70EX, N5 Premium - Inpout32.dll - _stdcall function | - S5, S10 - Atom_Control.dll (Atom_Control.dll file is copied with the system32 folder when Atom_IO_Control driver) - cdecl function |
| --- | --- |
| bool Set_WifiPower(short iPower); - Wifi On/Off - On : 1, Off : 0 bool Set_BluePower(short iPower); - Bluetooth On/Off - On : 1, Off : 0 bool Set_CameraPower(short iPower); - Camera On/Off - On : 1, Off : 0 bool Set_ModemPower(short iPower); - Modem On/Off - On : 1, Off : 0 bool Set_DmbPower(short iPower); - DMB On/Off - On : 1, Off : 0 | ULONG _ATOM_Power_On(UCHAR uPower); ULONG _ATOM_Power_Off(UCHAR uPower); - Wifi : 7 - BlueTooth : 2 - Camera : 3 (S10) - Modem : 1 - DMB : 6 - Internal Speaker Off : 4 (S10) - Headset : 5 (S10) |
| char Get_DevicePower(); - Device Power Status (Wifi, Bluetooth, Camera, Modem, DMB) - Display the binary |  |
| bool Set_BacklightLevel_ACK(short level); bool Set_BacklightLevel(short level); - Change the backlight brightness (0 ~ 15) -When first executing first, call Set_BacklightLevel_ACK. When second executing, Call Set_BacklightLevel. | ULONG _ATOM_Set_LCD_Backlit(UCHAR level); - Change the backlight brightness ( 0 ~ 15) |
| bool Set_PivotInfo(short rotation); - Send the Pivot status to the EC. - 0 : 0, 1 : 90, 2 : 180, 3 : 270 | ULONG _ATOM_Touch_Rotation(UCHAR rotation); - Send the Pivot status to the EC. - 0 : 0, 1 : 90, 2 : 180, 3 : 270 |
| bool Set_SoundOut(short iPower); - 1 : Speaker Out, 0 : Line Out - S7, X70EX |  |
| bool Set_KeyRemap(short iPower); - 0 : <Fn + Ctrl>, 1 : <Ctrl + Fn> - S7, X70EX |  |
| bool Haptic_Control(BYTE onoff, BYTE duration, BYTE mode); - Set the Vibration option. - X70EX onoff; duration; mode. Not Use; 0x55; 0x01 ~ 0xFF; 0x50. Only SoftKeyboard; 0xAA; 0x01 ~ 0xFF; 0x51. Always; 0xAA; 0x01 ~ 0xFF; 0x52. bool ExecuteSWHaptic(); - Execute the Viration. - X70EX | ULONG _ATOM_Haptic_Control(UCHAR on_off, UCHAR duration, UCHAR mode); - Set the Vibration option. - S5 onoff; duration; mode. Not Use; 0x55; 0x50. Only SoftKeyboard; 0xAA; 0x51. Always; 0xAA; 0x52. onoff; duration; mode. Execute; 0xAA; 0x01 ~ 0xFF; 0xAA. |